# Business Analysis

Requirements definition

## Mission Statement

Design and validate an elevator simulation tool that enables a Modeling Simulation & Testing team determine how many elevators are needed in a multi‑storey building to meet guest wait‑time objectives. The tool should replicate guest arrivals and elevator dispatch, measure actual and perceived wait times, and enable capacity planning decisions for parks and resorts.

Mission Overview: The mission of the Simple Elevator Simulation project is to develop a foundational simulation environment that models the flow of individual guests into and out of a 40-story building served by elevators. This system will simulate the behavior of elevator dispatch and passenger movement throughout a single operational day. The purpose is to provide actionable insight into wait times, system bottlenecks, and capacity thresholds for elevator usage in vertical facilities such as towers, resorts, or future-themed destinations.

This simulation serves as a prototype for future applications in theme park environments, where understanding the dynamics of guest surges and vertical transportation can inform infrastructure planning and attraction capacity strategies.

## System Value

By analyzing wait‑time distributions under different arrival rates, elevator counts and dispatch policies, AEC companies can size elevator systems so that less than 5 % of guests wait longer than one minute. This protects guest experience and throughput; the same methodology can later be adapted to attraction queues and park‑wide crowd‑flow analyses. Industry guidance suggests average elevator wait times in academic office buildings should not exceed 28–35 (SRC-01), and grade‑A office buildings aim for roughly 20 s (SRC-02). Hence setting a 95th‑percentile wait‑time target of 60 s provides a clear performance measure aligned with recognized service levels.

Value Proposition The elevator simulator provides a systematic and data-driven method to evaluate the impact of elevator quantity, dispatch strategy, and passenger arrival patterns on overall service efficiency. Specifically, the system will:

- Support throughput analysis for vertical transportation based on time-of-day usage.

- Determine the elevator count required to meet service-level goals.

- Quantify guest wait times and highlight conditions leading to degraded performance.

- Identify surge thresholds beyond which bottlenecks form, reducing guest satisfaction.

By modeling these elements, the simulator can aid in future planning of attraction queues, resort towers, and multi-level theme park environments. It supports scalable use for both conceptual design and operational testing in themed entertainment development.

## Initial Stakeholder Needs

| Stakeholder | Need/Concern | Notes |
| --- | --- | --- |
| Modeling Simulation & Testing team | A configurable tool to experiment with passenger arrival rates, elevator counts, dispatch logic and building height; ability to generate summary statistics and visualize system behavior. | Primary user of the simulation |
|  | Need to model and evaluate guest surge behaviors in vertical environments | 95% of passengers experience <1 min wait time |
| Global Engineering & Testing (GE&T) | Identify throughput bottlenecks for infrastructure planning | Maximum building throughput before service degradation |
| Park building designers & operations managers | Evidence-based recommendations on elevator count and control strategies to achieve specified wait-time objectives; insight into bottlenecks and surge scenarios | Secondary stakeholders |
| Imagineering Design & Planning | Understand simulation architecture for later reuse in themed attraction modeling | Modular architecture with configurable dispatch/pedestrian logic |
| Theme-park guests/occupants | Acceptable wait times and smooth elevator service (indirect stakeholder) | Research shows passengers become dissatisfied when waiting exceeds what they consider reasonable; quality of service is largely judged on average and perceived wait times |
| Corporate leadership | Ability to justify capital expenditures on elevators and throughput improvements based on quantified benefits | Alignment with business goals |

## Goals, objectives and performance measures

High-level Objectives

- Model elevator dispatch and guest traffic through a 40-story building across a full day (12:01am to 11:59pm) of simulated time.

- Simulate three distinct passenger classes (Residents, Office Workers, Shoppers), each with arrival time patterns and floor destination preferences.

- Allow configuration of:

  - Number of elevators

  - Arrival frequency for each passenger class

  - Maximum passenger throughput targets

  - Ensure that 95% of passengers wait less than 1 minute for an elevator throughout the simulated day.

- Capture logs of elevator actions and passenger wait/travel times to assess system performance.

- Create a simulation framework where Elevator and Passenger behavior are decoupled for future extensibility.

Goals

- Goal 1: Determine required elevator capacity.

  - Evaluate how many elevators are needed to keep the 95th percentile wait time below one minute and maintain average wait times close to recommended levels (20–35) (SRC-02).

- Goal 2: Analyse impact of arrival patterns and guest classes.

  - Model pedestrian traffic through bottleneck situations with multiple user types with different peak periods; test variable arrival rates and dwell times to understand sensitivity.

- Goal 3: Provide decision-support metrics

  - For each scenario, output distributions of wait times, travel times and throughput; report the percentage of passengers exceeding threshold waits; and compute utilisation of each vehicle (elevator).

- Goal 4: Support future adaptation

  - Develop the core architecture so that it can later incorporate destination dispatch, mid-journey floor changes, and eventually theme-park attraction queues.

## High-Level system requirements

These requirements shall be captured in the RTVM and later translated to SysML requirement diagrams after the mission and stakeholder analysis.

- Functional requirements

  - The simulator shall model individual passengers with arrival time, origin/destination floor, trip purpose and departure time.

  - It shall simulate elevator motion between floors, door operation, and passenger boarding/alighting.

  - It shall log events (calls, assignments, movements) in a machine‑readable file.

  - It shall allow configuration of building height, number of elevators and passenger arrival distributions.

- Performance requirements

  - The simulation shall produce wait‑time statistics.

  - It shall support experiments to verify that the 95th‑percentile wait is ≤ 60 s and report average wait times (expected 20–45 s based on building type guidelinesappa.org).

- Interface requirements

  - Initial implementation will be console‑based with JSON configuration (INPUT) and CSV/JSON logs (OUTPUT).

  - The architecture shall decouple passenger management from elevator dispatch so that dispatch strategies can be swapped without altering passenger logic.

  - Communication between passenger and elevator controllers shall be event‑based (delegates or messaging) to ensure loose coupling.

- Quality requirements

  - The simulator shall be deterministic given a fixed random seed.

  - It shall be able to run a full day (24 h) within minutes of computing time.

  - It shall record reproducible results for analysis.

## Success Criteria

These requirements shall be captured in the RTVM and later translated to SysML requirement diagrams after the mission and stakeholder analysis.

- Simulation runs a full operational day and outputs detailed logs.

- Data confirms that with optimal elevator count and distribution strategy, 95% of passengers meet the 1-minute wait time target.

- Model identifies inflection points where passenger surges or elevator shortages cause service degradation.

- Simulation engine architecture supports modular expansion for future GUI or dynamic dispatch logic.